AI Talking Toy for Kids (India)

Voice (TTS) Strategy & Cost Breakdown

Prepared: 18 July 2026  |  Currency: INR (₹)  |  Figures are working estimates

Decision: Do NOT run TTS on-device (edge) for launch

The on-device (edge) TTS model is small in file size (~60MB), so storage is NOT the problem. The problem is the SILICON it needs: Sarvam Edge is tuned for phone-class NPUs (Snapdragon Hexagon / MediaTek Dimensity) and 4–6 GB RAM. Putting that into every toy means a more expensive chip, more RAM, more battery — the exact power-usage and hardware-cost problem we want to avoid.

RECOMMENDED PATH: keep TTS in the cloud, and cache the repeated phrases as pre-made audio files. This cuts cost 40–60% with cheap hardware, minimal power, and no on-device AI.

1.  Why We Are Skipping Edge (the size vs. hardware truth)

Model file is tiny — 24M parameters, ~60MB on disk. A non-issue; even a cheap toy has room.

But it wants phone-grade hardware — Sarvam Edge is validated on Snapdragon Hexagon / MediaTek Dimensity NPUs and targets 4–6 GB RAM. That is mid-range-phone silicon. In a toy = higher chip cost + more RAM + bigger battery.

Power, honestly — TTS runs in short bursts (1–2 sec per reply), not continuously, so battery drain is modest. The real penalty is the pricier SoC, not the power.

Low-end chips can’t run it — A bare-bones toy MCU (ESP32-class) simply cannot run the Edge model in real time.

Conclusion: edge saves recurring cost but forces expensive hardware into every unit. Not worth it for a first product.

2.  The Smart Middle Path — Cloud TTS + Cached Playback

60–70% of a toy’s speech is REPEATED: greetings, praise (“Great job!”), songs, story intros, game prompts, error lines. Strategy:

Generate those repeated lines ONCE with cloud Bulbul TTS.

Store them as audio files on cheap flash. The toy just PLAYS THEM BACK — near-zero compute, near-zero power, works even offline.

Only call cloud TTS for the truly dynamic replies from the LLM brain.

Result: 40–60% lower TTS cost, with NO NPU, NO extra RAM, NO power hit — just cheap flash storage.

3.  Approach Comparison

| Approach | Hardware cost | Power | Recurring TTS cost | Verdict |
| --- | --- | --- | --- | --- |
| Full Edge TTS | High (phone-grade SoC + 4–6GB RAM) | Low-ish (bursts) | ₹0 | Overkill now |
| Cloud + cached playback | Cheap | Minimal | ~40–60% lower | BEST for launch |
| Pure cloud TTS | Cheap | Minimal | Full (₹0.30/turn) | Simple but pricey |

4.  Cost Breakdown (Recommended: Cloud + Cached)

A “turn” = child speaks once (~10 sec) + toy replies once. Assume ~20 turns/day for an engaged child, and ~65% of replies served from cached audio (₹0).

| Step | Pure cloud | Cloud + cached (recommended) |
| --- | --- | --- |
| STT (Saaras, cloud) | ₹0.08 | ₹0.08 |
| LLM brain (Sarvam-30B, cloud) | ₹0.005 | ₹0.005 |
| TTS (voice) | ₹0.30 | ~₹0.11 (only 35% hit cloud) |
| TOTAL PER TURN | ₹0.40 | ~₹0.19 |

| Per active toy | Pure cloud | Cloud + cached (recommended) |
| --- | --- | --- |
| Per day (20 turns) | ₹8.00 | ~₹3.90 |
| Per month | ~₹240 | ~₹117 |
| Per year | ~₹2,880 | ~₹1,400 |

Caching roughly halves recurring cost while keeping the hardware cheap and simple.

5.  Hardware — Cheap & Simple (no NPU needed)

| Component | Estimate |
| --- | --- |
| SoC (basic, WiFi, audio) — no NPU needed | ₹800–1,500 |
| Mic (for clear kid speech) | ₹150–400 |
| Speaker + amp | ₹150–400 |
| Flash storage (for cached audio) | ₹100–300 |
| Battery | ₹200–400 |
| PCB, enclosure, assembly, misc | ₹700–1,800 |
| TOTAL BOM (est.) | ₹2,100–4,800 |

No phone-grade NPU, no 4–6GB RAM — this is standard cheap IoT-toy hardware.

6.  Bottom Line

Skip edge for launch: the edge model is small (60MB), but it demands phone-grade silicon — so it forces expensive hardware and power into every toy.

Do this instead: cloud TTS + cached playback → ~40–60% lower recurring cost, cheap hardware, minimal power.

Later: watch real per-toy usage. Revisit on-device TTS at scale, when cost justifies it and Edge runs on cheaper chips.

Appendix — Sarvam Edge (On-Device) Reference

Kept for future reference if on-device is reconsidered at scale.

Edge TTS model specs

24M parameters, ~60MB on disk

10 Indian languages, 8 voices from one compact model

~260ms to start speaking; 5× faster than real-time (Samsung Galaxy S25 Ultra)

Validated on Snapdragon Hexagon / MediaTek Dimensity NPUs; targets 4–6 GB RAM

Sustained multi-minute inference can cause thermal throttling (not relevant for burst TTS)

Edge economics (if ever used)

Recurring TTS cost → ₹0 (runs offline on device)

BUT adds ~₹1,500–3,000/unit hardware for the NPU-class SoC + RAM

Sarvam Edge is a commercial product (not a public API) — license price must be quoted by Sarvam; could be per-device royalty or flat SDK fee

Open-source fallback: Piper TTS — ₹0 license, runs on Raspberry Pi 5, slightly lower quality

Sources

Sarvam Edge (official): sarvam.ai/products/edge

Sarvam Edge details: analyticsvidhya.com/blog/2026/03/sarvam-edge

Sarvam models & pricing: docs.sarvam.ai/api/getting-started/models  |  sarvam.ai/api-pricing

Piper TTS (offline fallback): localaimaster.com/blog/piper-tts-setup-guide

Caveats

BOM figures are industry estimates — get real quotes from a hardware manufacturer.

API prices change — verify current rates on Sarvam’s pricing page before finalizing.

Kids’ product = higher bar: budget for content-safety filtering and DPDP Act compliance (parental consent, no ad-targeting to children).